# Condition Assessment Report

25474-SECURITY FORCES

N. Beale Rd.

Beale AFB, CA 95901

| ### Inspection Date: | Monday, June 26, 2017 |
| --- | --- |

# Building & Job Summary

| Building Name | 25474-SECURITY FORCES |
| --- | --- |
| Building Address | N. Beale Rd. Beale AFB, CA 95901 |
| Year Built | 1996 |
| Roof Area (total) | 5,919 SF |
| Building Description | 25474-SECURITY FORCES Building Type: Institutional/Government Building Zone: Institutional |
| Inspection Date | Monday, June 26, 2017 |
| Inspector | Beale AFB Clark Peterson clark.peterson.2@us.af.mil (P) 530-392-2431 |
| Inspection Notes | Needs recoat... WO# 2576977 Recoated with armorgarage 8/16/2017 |
| Building Contact | Notes: nicolas.peterson.1@us.af.mil danielle.yared@us.af.mil raymond.rocheleau@us.af.mil |
| Client Contact |  |

# Section Key Plan

# Section Information

| Section ID: A Area: 5,919 SF Roof Type: Metal Layers: ● Metal surface: ○ Material: Steel ○ Type: Structural panel ○ Profile: Standing seam ○ Color: Brown Roof Condition Score: 100 (Adequate) |  |
| --- | --- |

# Roof Condition Scores

|  | Section |  | Score |  |
| --- | --- | --- | --- | --- |
|  | A (Area = 5,919 SF) |  | 100 - Adequate |  |
| Legend |  |  |  |  |
|  | Adequate ( 100% ) |  |  |
|  | Degraded ( 0% ) |  |  |
|  | Unsatisfactory ( 0% ) |  |  |
|  | Unspecified ( 0% ) |  |  |

# Section A

## Section Summary – A

Section ID:	A

Area:	5,919 SF

Year Installed:	2002 (approximate)

Height:	20 FT

Floors:	0

Sensitivity:	High

Roof Condition Score:	100 (Adequate)

Roof Type:  Metal

Layers:

● Metal surface:

○ Material: Steel

○ Type: Structural panel

○ Profile: Standing seam

○ Color: Brown

Notes:  Prior RCS=55; prior ERSL React/Proact=15/1. Prior RCS=55; prior ERSL React/Proact=15/1.

## Inventory Detail – Section A

| Segment ● Type: Edge |  |
| --- | --- |
| Drip edge ● Gutter: Yes |  |
| Metal flue ● Diameter: 4" ● Flashing: Pipe jack |  |
| Metal flue ● Diameter: 4" ● Flashing: Pipe jack |  |
| Metal flue ● Diameter: 4" ● Flashing: Pipe jack |  |
| Metal flue ● Diameter: 4" ● Flashing: Pipe jack |  |
| Metal flue ● Diameter: 4" ● Flashing: Pipe jack |  |
| Metal flue ● Diameter: 4" ● Flashing: Pipe jack |  |
| Plumbing vent ● Size: 3" ● Material: Cast iron ● Flashing: Rubber boot |  |
| Plumbing vent ● Size: 2" ● Material: Cast iron ● Flashing: Rubber boot |  |

## Roof Condition Score – Section A

| # | Question (Max Value) / Notes | Score |
| --- | --- | --- |
| 1 | Roof is less than 5 years old and in excellent condition - no observed deficiencies (0) |  |
| 2 | Are there roof leaks? (5) |  |
| 3 | Have the past roof leaks caused significant interior damage? (10) |  |
| 4 | Have roof leaks caused significant damage to building's structural integrity? (10) |  |
| 5 | Has the roof been leaking for over 1 year? (7) |  |
| 6 | Does the roof leak every time it rains in 3 or fewer places? (5) |  |
| 7 | Does the roof leak every time it rains in 4 or more places? (10) |  |
| 8 | Is the surface of the roofing deteriorated excessively? (4) |  |
| 9 | Is there excessive patching on the roof? (8) |  |
| 10 | Is roof-top equipment mounted improperly? (5) |  |
| 11 | Is roof installation workmanship of medium quality? (3) |  |
| 12 | Is roof installation workmanship of poor quality? (5) |  |
| 13 | Remaining roof life > 75% (5) |  |
| 14 | Remaining roof life 50% - 74% (10) |  |
| 15 | Remaining roof life 25% - 49% (20) |  |
| 16 | Remaining roof life <25% (30) |  |
| 17 | Does the flashing look deteriorated and appear to be letting the water enter the building in a few places? (6) |  |
| 18 | Does the flashing look deteriorated and appear to be letting the water enter the building in an excessive number of places? (10) |  |
| 19 | Is exposed sealant showing signs of moderate deterioration? (3) |  |
| 20 | Is exposed sealant showing signs of excessive deterioration? (5) |  |
| 21 | Is there a moderate number of metal coating defects? (3) |  |
| 22 | Is there an excessive number of metal coating defects? (5) |  |
| 23 | Is there a moderate number of loose or missing fasteners? (3) |  |
| 24 | Is there an excessive number of loose or missing fasteners? (5) |  |
| 25 | Is there a moderate number of end lap defects (eave termination/transverse joints)? (3) |  |
| 26 | Is there an excessive number of end lap defects (eave termination/transverse joints)? (5) |  |
| 27 | Are the joints in the metal open to let water enter? (2) |  |
| 28 | Is sheet metal loose and or missing? (3) |  |
| 29 | Miscellaneous and other conditions - Low (5) |  |
| 30 | Miscellaneous and other conditions - Medium (10) |  |
| 31 | Miscellaneous and other conditions - High (15) |  |
| 32 | Roof is at the end of its life - Immediate Action Required! (0) |  |
|  | Total | 0 |
|  | FINAL SCORE (100 – Total) | 100 |
|  | ROOF CONDITION SCORE: Adequate |  |

# Photo Album

| Building Photo | Building Photo |
| --- | --- |
| Inspection/Job Photo: Preasure washing 1 | Inspection/Job Photo: Preasure washing 2 |
| Inspection/Job Photo: Primmer | Inspection/Job Photo: Base coat 1 |
| Inspection/Job Photo: Base coat 2 | Inspection/Job Photo: Base coat 3 |
| Inspection/Job Photo: Base coat 4 | Inspection/Job Photo: top coat 1 |
| Inspection/Job Photo: top coat 2 | Inspection/Job Photo: top coat 3 |
| Inspection/Job Photo: top coat 4 | Inspection/Job Photo: complete |